SECTION 10503

WINDSOR OUTDOOR HOSE DRYER

STORAGE UNIT

# PART 1 GENERAL

  - SECTION INCLUDES

- Design and fabrication of Windsor Outdoor Hose Dryer unit as specified herein.

  - RELATED SECTIONS

A.   N/A

  - SUBMITTALS

- Product Data: Submit manufacturer’s product data and installation instructions.

- Shop Drawings: Submit manufacturer’s shop drawings for each unit.

- Samples: Submit manufacturer’s standard color samples.

- Owner’s Manual: Provide maintenance manual at closeout.

- Warranty: Submit manufacturer’s standard warranty.

  - QUALITY ASSURANCE

- Manufacturer shall have a minimum of five years experience in the direct manufacture of fire market equipment handling and storage components.

- Installer shall have experience in fire market equipment handling and storage component assembly and installation.

  - DELIVERY, STORAGE AND HANDLING

A: Delivery: Deliver materials to site in manufacturer’s original, unopened containers with labels identifying product and manufacturer’s name.

B: Storage: Store materials in a clean dry area.

C: Handling: Protect materials and finish during installation and handling to prevent damage.

### PART 2 PRODUCTS

2.1 MANUFACTURER

- GearGrid Corporation, 670 SW 15th Street, Forest Lake, MN 55025. Toll-free 888-643-6694. Phone 651-464-4468. Fax 651-464-4780. Web site www.geargrid.com. Email sales@geargrid.com.

2.2 HOSE DRYER SYSTEM

A.   Model:

1. GEARGRID Windsor Outdoor Hose Dryer

a. Unit overall dimensions: 36.5” high x 60” wide x 48’ long.

B.   Functionality:

1. For the proper internal and external drying of Fire Hose after use or washing. Shall accommodate sections of hose up to 50’ in length, with a continuous downward 12” slope of the system, to promote gravity draining of the entire length of hose and to prevent trap areas where water can collect internally.

B.   Construction:

1. Units shall be welded at all applicable joints. Forming of metal shall be completed by standard              cold-forming operations. Use of fasteners will only be required to allow for knock-down shipping and for securing units to mounting surface.

C.   Vertical Framework:

1. Outer Frames: Heavy duty 1-1/2” steel tubing supports and 1-1/4” frames of ASTM A513 steel tubing.

## D.   Hose Dryer Grid:

1. Grid: High strength .25” diameter ASTM 510 cold drawn steel wire resistance welded to a rectilinear pattern.

2.3 FINISH

A. General: All system components excluding assembly and mounting hardware and stainless steel components are to receive the standard finish.

B. Standard Finish: Components to be cleaned using a phosphatized bath, clear water rinse and electro-statically coated with an Epoxy primer with durable TGIC powder top coating.

C. Color: _______________

Part 3 EXECUTION

3.1 EXAMINATION

A. Examine areas to receive Hose Dryer. Notify architect if areas are not acceptable. Do not begin installation until unacceptable conditions have been corrected.

3.2 INSTALLATION

A. Assemble Windsor hose dryer in accordance with manufacturer’s instructions.

B. Use manufacturer’s hardware for assembly.

C. Anchor to mounting surface with proper hardware.

- Mounting hardware supplied by installer/owner

END OF SECTION