SECTION 10503

HEAVY DUTY MOBILE HOSE/SCBA BOTTLE

STORAGE UNITS

# PART 1 GENERAL

  - SECTION INCLUDES

- Design, fabrication and installation of mobile hose/scba storage units as specified herein.

  - RELATED SECTIONS

A.   Section 10: Metal Lockers

  - SUBMITTALS

- Product Data: Submit manufacturer’s product data and installation instructions.

- Shop Drawings: Submit manufacturer’s shop drawings for each mobile hose/scba storage units.

- Samples: Submit manufacturer’s standard color samples.

- Owner’s Manual: Provide maintenance manual at closeout.

- Warranty: Submit manufacturer’s standard warranty.

  - QUALITY ASSURANCE

- Manufacturer shall have a minimum of five years experience in the direct manufacture of lockers and storage systems.

- Installer shall have experience in locker assembly and installation.

  - DELIVERY, STORAGE AND HANDLING

A: Delivery: Deliver materials to site in manufacturer’s original, unopened containers with labels identifying product and manufacturer’s name.

B: Storage: Store materials in a clean dry area.

C: Handling: Protect materials and finish during installation and handling to prevent damage.

### PART 2 PRODUCTS

2.1 MANUFACTURER

- Mid-Minnesota Wire (GearGrid Product Line), 670 SW 15th Street, Forest Lake, MN 55025. Toll-free 888-643-6694. Phone 651-464-4468. Fax 651-464-4780. Web site www.geargrid.com. Email sales@geargrid.com.

2.2 STORAGE SYSTEMS

A.   Models:

1. GEARGRID Heavy-Duty Hose/SCBA Unit/Three Openings Wide.

a. Unit overall dimensions: 83” high x 75” wide x 26” deep.

b. Clear opening width: 22.75”

C. Construction:

1. Units shall be welded at all applicable joints. Forming of metal shall be completed by standard              cold-forming operations. Use of fasteners will only be required to allow for knock-down shipping, securing units to mounting surface and on applicable accessories.

D. Vertical Dividers:

1. Outer Frames: 1.25” O.D. x 16 gauge wall thickness ASTM A513 steel tubing.

2. Inner Grid: .25” diameter ASTM 510 cold drawn steel wire resistance welded to a 3” square pattern.

E. Back Panel:

1. Grid: .25” diameter ASTM 510 cold drawn steel wire resistance welded to a 3” square pattern.

## F: Hose Storage Shelves:

## 1. Shelves shall be manufactured from1.25” x 16 gauge wall thickness square ASTM A513 steel tubing, ¼ x1-1/2 ASTM A36 steel bar and .375” diameter ASTM A510 steel round. Steel surface finish to allow application of durable powder coat finish.

G: SCBA Bottle Storage Shelves:

- 1.Shelves shall be manufactured from .25” diameter ASTM A510                                                                                                                                                                                                                               cold drawn steel wire resistance welded. All bends to be cold formed. All areas which contact bottles are to be smooth and/or rounded to prevent marring of bottle. Wire surface to allow application of durable powder coated finish.

H. Base Assembly:

1. Base frame shall be manufactured from 1.25” x 11 gauge wall thickness ASTM

A513 square steel tubing. Each unit to be supplied with four (4) casters per unit.

2. Caster Capacity:

a. HD Hose/SCBA Unit: 950 lbs. each caster.

2.3 FINISH

A. General: All system components excluding assembly and mounting hardware and stainless steel components are to receive the standard finish.

B. Standard Finish: Components to be cleaned using a phosphatized bath, clear water rinse and electro-statically coated with a durable TGIC powder coating.

C. Color: _______________

Part 3 EXECUTION

3.1 EXAMINATION

A. Examine areas to receive lockers. Notify architect if areas are not acceptable. Do not begin installation until unacceptable conditions have been corrected.

3.2 INSTALLATION

A. Assemble mobile lockers in accordance with manufacturer’s instructions.

B. Use manufacturer’s hardware for assembly.

END OF SECTION